我模拟整个卫星过境，你可以选择时间槽15卫星的位置，坐标单位是m，此时卫星在目标区域的正上方。

def _initialize_satellite_trajectory(self):

"""初始化卫星轨迹数据"""

# 卫星轨迹数据

self.satellite_positions = [

[-1556838.79, 1743703.31, 421313.65],  # 时间槽1

[-1456353.69, 1625072.54, 472941.46],  # 时间槽2

[-1355328.81, 1505409.94, 521013.57],  # 时间槽3

[-1253810.96, 1384783.08, 565504.83],  # 时间槽4

[-1151847.12, 1263260.10, 606391.96],  # 时间槽5

[-1049484.35, 1140909.72, 643653.60],  # 时间槽6

[-946769.78, 1017801.18, 677270.30],  # 时间槽7

[-843750.63, 894004.19, 707224.55],  # 时间槽8

[-740474.13, 769588.88, 733500.75],  # 时间槽9

[-636987.55, 644625.80, 756085.27],  # 时间槽10

[-533338.14, 519185.85, 774966.40],  # 时间槽11

[-429573.13, 393340.23, 790134.43],  # 时间槽12

[-325739.72, 267160.45, 801581.57],  # 时间槽13

[-221885.01, 140718.20, 809302.04],  # 时间槽14

[-118056.04, 14085.41, 813291.98],  # 时间槽15

[-14299.72, -112665.88, 813549.56],  # 时间槽16

[89337.15, -239463.49, 810074.88],  # 时间槽17

[192807.93, -366235.21, 802870.04],  # 时间槽18

[296066.17, -492908.77, 791939.09],  # 时间槽19

[399065.59, -619411.94, 777288.06],  # 时间槽20

[501760.13, -745672.56, 758924.96],  # 时间槽21

[604103.96, -871618.55, 736859.73],  # 时间槽22

[706051.51, -997177.99, 711104.30],  # 时间槽23

[807557.47, -112279.13, 681672.52],  # 时间槽24

[908576.85, -1246850.48, 648580.20],  # 时间槽25

[1009064.94, -1370820.78, 611845.08],  # 时间槽26

[1108977.38, -1494119.11, 571486.80],  # 时间槽27

[1208270.15, -161674.90, 527526.94],  # 时间槽28

[1306899.61, -1738417.95, 479988.96],  # 时间槽29

[1404822.50, -1859278.52, 428898.20],  # 时间槽30

]

用户的位置是以（0，0，0）为圆心的半径为15000m的圆。也就是你在（0，15000，0）（0，-15000，0）（-15000，0，0）(15000，0，0)里选点就可以

要注意，两跳链路，第二跳的最终速率是取第一跳和第二跳速率的最小值

卫星到地面或者无人机速率计算：

发射功率：卫星:20W, 无人机:（3-5w）,UE:0.2w

卫星到设备的频率：20GHz

卫星每信道带宽：20MHZ(也可以调整)

卫星天线增益：32dBi

无人机天线增益：25dBi

用户天线增益：5dBi

hsat,u(t)=c4πfsatdsat,u(t)2GsatGuδsat,u(t)Su(t),

import numpy as np

import math

from scipy.special import jv

C = 3e8

FREQ = 20e9

G_SAT_TX_LINEAR = 10 ** (32.0 / 10)

G_USER_RX_LINEAR = 10 ** (5.0 / 10)

G_UAV_RX_LINEAR = 10 ** (25.0 / 10)

BEAM_ANGLE_B_DEG = 0.85

BEAM_UK_N_COEFF = 2.07123

NOISE_DENSITY_W_PER_HZ = 10 ** ((-174 - 30) / 10)

def beam_pattern_single_beam(user_pos_3d, sat_pos_3d):

beam_center_3d = np.zeros(3)

vec_sat_to_beam = beam_center_3d - sat_pos_3d

vec_sat_to_user = np.array(user_pos_3d) - sat_pos_3d

norm_beam = np.linalg.norm(vec_sat_to_beam)

norm_user = np.linalg.norm(vec_sat_to_user)

if norm_beam == 0 or norm_user == 0:

return 0.0

cos_phi = np.clip(np.dot(vec_sat_to_beam, vec_sat_to_user) / (norm_beam * norm_user), -1.0, 1.0)

phi_rad = np.arccos(cos_phi)

if phi_rad < 1e-9:

return 1.0

sin_B = math.sin(math.radians(BEAM_ANGLE_B_DEG))

if abs(sin_B) < 1e-9:

return 1e-10

uk_n = BEAM_UK_N_COEFF * math.sin(phi_rad) / sin_B

if abs(uk_n) < 1e-9:

return 1.0

term1 = jv(1, uk_n) / (2 * uk_n)

term2 = 36 * jv(3, uk_n) / (uk_n ** 3)

return float(np.clip((term1 + term2) ** 2, 1e-10, 1.0))

def effective_gain(sat_pos, dev_pos, is_uav):

distance = np.linalg.norm(dev_pos - sat_pos)

distance = max(distance, 1e-9)

path_loss_inv = (C / (4 * math.pi * distance * FREQ)) ** 2

beam_gain = beam_pattern_single_beam(dev_pos, sat_pos)

G_rx = G_UAV_RX_LINEAR if is_uav else G_USER_RX_LINEAR

return path_loss_inv * G_SAT_TX_LINEAR * G_rx * beam_gain

def oma_rate_single_device(sat_pos, dev_pos, P_tx_W, B_Hz, is_uav=False):

gain = effective_gain(sat_pos, dev_pos, is_uav)

signal_W = P_tx_W * gain

noise_W = NOISE_DENSITY_W_PER_HZ * B_Hz

sinr = signal_W / noise_W

rate_mbps = B_Hz * np.log2(1 + sinr) / 1e6

return rate_mbps, sinr, gain

def noma_rate_two_devices(sat_pos, weak_pos, strong_pos,

P_tx_W, B_Hz, alpha_power,

weak_is_uav=False, strong_is_uav=False):

gain_weak = effective_gain(sat_pos, weak_pos, weak_is_uav)

gain_strong = effective_gain(sat_pos, strong_pos, strong_is_uav)

noise_W = NOISE_DENSITY_W_PER_HZ * B_Hz

signal_weak = alpha_power * P_tx_W * gain_weak

interference_weak = (1 - alpha_power) * P_tx_W * gain_weak

sinr_weak = signal_weak / (interference_weak + noise_W)

signal_strong = (1 - alpha_power) * P_tx_W * gain_strong

sinr_strong = signal_strong / noise_W

rate_weak = B_Hz * np.log2(1 + sinr_weak) / 1e6

rate_strong = B_Hz * np.log2(1 + sinr_strong) / 1e6

return (rate_weak, rate_strong), (sinr_weak, sinr_strong), (gain_weak, gain_strong)

用法示例：

import numpy as np

# 假设：卫星在 GEO 上方

sat = np.array([0.0, 0.0, 35786e3])  # m

# 地面用户 A、B（z=0），UAV 在 1km 高空

user_A = np.array([5e3, 2e3, 0.0])

user_B = np.array([-4e3, 3e3, 0.0])

uav = np.array([3e3, -1e3, 1e3])

B = 20e6       # 单信道带宽 20 MHz

P = 5.0        # 单信道发射功率 5 W

alpha = 0.7    # NOMA 分配：弱用户 70% 功率（NOMA原理就是给远端用户也就是弱用户分配更高的功率，这个也可以设置成0.8）

# --- OMA 示例：用户 A ---

rate_A, sinr_A, gain_A = oma_rate_single_device(sat, user_A, P, B, is_uav=False)

print(f"OMA 用户A: {rate_A:.3f} Mbps, SINR={10*np.log10(sinr_A):.2f} dB, gain={10*np.log10(gain_A):.2f} dB")

# --- OMA 示例：UAV ---

rate_uav, sinr_uav, gain_uav = oma_rate_single_device(sat, uav, P, B, is_uav=True)

print(f"OMA UAV: {rate_uav:.3f} Mbps, SINR={10*np.log10(sinr_uav):.2f} dB, gain={10*np.log10(gain_uav):.2f} dB")

# --- NOMA 示例：用户 A (弱) + UAV (强) ---

(rates, sinrs, gains) = noma_rate_two_devices(

sat, weak_pos=user_A, strong_pos=uav,

P_tx_W=P, B_Hz=B, alpha_power=alpha,

weak_is_uav=False, strong_is_uav=True

)

rate_weak, rate_strong = rates

sinr_weak, sinr_strong = sinrs

gain_weak, gain_strong = gains

print(f"NOMA 弱用户A: {rate_weak:.3f} Mbps, SINR={10*np.log10(sinr_weak):.2f} dB")

print(f"NOMA 强UAV: {rate_strong:.3f} Mbps, SINR={10*np.log10(sinr_strong):.2f} dB")

无人机到地面用户终端计算速率，

无人机发射功率（3-5w）

无人机或者UE中继到UE的频率：2.4GHz

无人机和UE中继每信道带宽：2MHz（也可以调整）

无人机高度我设置的是1000m

hn,m(t)=c4πfagdn,m(t)2ℒn,m(t),

import numpy as np

C = 3e8

FREQ_UAV = 2.4e9

A_SUB, B_SUB = 4.88, 0.43

ETA_LOS_DB, ETA_NLOS_DB = 1.0, 21.0

N0_W_PER_HZ = 10 ** ((-174 - 30) / 10)

def uav_to_user_gain(uav_pos, user_pos):

"""

uav_pos, user_pos: np.array([x, y, z]) in meters

return: 信道增益 (线性值)

"""

horiz_dist = np.linalg.norm(uav_pos[:2] - user_pos[:2])

height = max(uav_pos[2] - user_pos[2], 1e-9)

theta_deg = 90.0 if horiz_dist <= 0 else np.degrees(np.arctan(height / horiz_dist))

P_los = 1.0 / (1.0 + A_SUB * np.exp(-B_SUB * (theta_deg - A_SUB)))

d_slant = np.sqrt(height ** 2 + horiz_dist ** 2)

fspl_dB = 20 * np.log10(d_slant) + 20 * np.log10(FREQ_UAV) + 20 * np.log10(4 * np.pi / C)

excess_dB = P_los * ETA_LOS_DB + (1 - P_los) * ETA_NLOS_DB

total_dB = fspl_dB + excess_dB

return (10 ** (-total_dB / 10.0))

def uav_oma_rate(uav_pos, user_pos, P_uav_W, B_Hz):

gain = uav_to_user_gain(uav_pos, user_pos)

signal = P_uav_W * gain

noise = N0_W_PER_HZ * B_Hz

sinr = signal / noise

rate_mbps = B_Hz * np.log2(1 + sinr) / 1e6

return rate_mbps, sinr, gain

def uav_noma_rate(uav_pos, weak_user_pos, strong_user_pos,

P_uav_W, B_Hz, alpha_power):

gain_w = uav_to_user_gain(uav_pos, weak_user_pos)

gain_s = uav_to_user_gain(uav_pos, strong_user_pos)

noise = N0_W_PER_HZ * B_Hz

sig_w = alpha_power * P_uav_W * gain_w

int_w = (1 - alpha_power) * P_uav_W * gain_w

sinr_w = sig_w / (int_w + noise)

sig_s = (1 - alpha_power) * P_uav_W * gain_s

sinr_s = sig_s / noise  # 强用户 SIC 后仅剩噪声

rate_w = B_Hz * np.log2(1 + sinr_w) / 1e6

rate_s = B_Hz * np.log2(1 + sinr_s) / 1e6

return (rate_w, rate_s), (sinr_w, sinr_s), (gain_w, gain_s)

用法示例：

uav = np.array([2000.0, 1000.0, 1200.0])   # UAV 位置

user1 = np.array([0.0, 0.0, 0.0])

user2 = np.array([2500.0, 500.0, 0.0])

P_uav = 3.16      # 35 dBm ≈ 3.16 W

B = 2e6          # 12 MHz

alpha = 0.7

rate1, sinr1, gain1 = uav_oma_rate(uav, user1, P_uav, B)

(rates, sinrs, gains) = uav_noma_rate(

sat_uav, weak_user_pos=user1, strong_user_pos=user2,

P_uav_W=P_uav, B_Hz=B, alpha_power=alpha

)

UE和UE之间计算速率

用户发射功率0.2w

带宽2MHz

import numpy as np

C = 3e8

FREQ_D2D = 2.4e9

PATH_LOSS_EXP = 3.0

REF_DIST = 1.0

N0_W_PER_HZ = 10 ** ((-174 - 30) / 10)

def d2d_channel_gain(user_a_pos, user_b_pos):

"""用户-用户 D2D 信道增益 (线性值)"""

distance = np.linalg.norm(np.array(user_a_pos) - np.array(user_b_pos))

if distance <= 0:

return 0.0

fspl_d0_dB = 20 * np.log10(REF_DIST) + 20 * np.log10(FREQ_D2D) + 20 * np.log10(4 * np.pi / C)

if distance > REF_DIST:

path_loss_dB = fspl_d0_dB + 10 * PATH_LOSS_EXP * np.log10(distance / REF_DIST)

else:

path_loss_dB = 20 * np.log10(distance) + 20 * np.log10(FREQ_D2D) + 20 * np.log10(4 * np.pi / C)

path_gain_linear = 10 ** (-path_loss_dB / 10.0)

return path_gain_linear

def d2d_oma_rate(user_a_pos, user_b_pos, P_tx_W, B_Hz):

"""单用户 D2D OMA 速率 (Mbps)"""

gain = d2d_channel_gain(user_a_pos, user_b_pos)

signal = P_tx_W * gain

noise = N0_W_PER_HZ * B_Hz

sinr = signal / noise

rate_mbps = B_Hz * np.log2(1 + sinr) / 1e6

return rate_mbps, sinr, gain

def d2d_noma_rate(tx_pos, weak_rx_pos, strong_rx_pos,

P_tx_W, B_Hz, alpha_power):

"""

同一发射端对两个用户做 NOMA，下行速率 (弱, 强)

tx_pos: 发射端坐标

weak/strong_rx_pos: 两个接收端坐标

alpha_power: 分给弱用户的功率份额

"""

gain_w = d2d_channel_gain(tx_pos, weak_rx_pos)

gain_s = d2d_channel_gain(tx_pos, strong_rx_pos)

noise = N0_W_PER_HZ * B_Hz

sig_w = alpha_power * P_tx_W * gain_w

int_w = (1 - alpha_power) * P_tx_W * gain_w

sinr_w = sig_w / (int_w + noise)

sig_s = (1 - alpha_power) * P_tx_W * gain_s

sinr_s = sig_s / noise  # 强用户 SIC

rate_w = B_Hz * np.log2(1 + sinr_w) / 1e6

rate_s = B_Hz * np.log2(1 + sinr_s) / 1e6

return (rate_w, rate_s), (sinr_w, sinr_s), (gain_w, gain_s)